aNGLO oPERATIONS lIMITED

spectrem air Limited

NOVEMBER 2005

Keywords

REPORT of the SPECTREM SURVEY

LAC BELLEAU PROJECT AREA

for

ANGLO AMERICAN EXPLORATION (CANADA) LTD

Lac Belleau, Canada, Anglo American Exploration (Canada) Ltd, SPECTREM, Airborne, Electromagnetic, Magnetic

SUMMARY

The Spectrem Airborne Electromagnetic Survey over the Lac Belleau Area mapped number of conductors within the surveyed area.  These are mainly situated within a belt of basalt which extends in a north westerly direction through the area

Eight anomalous zones have been identified. The zones have been graded as low priority but would require some form of ground investigation.

- CONTRIBUTORS

Philip Klinkert:

Louis Polome:

- CIRCULATION LIST

Copies of this report have been sent to:

- Mr.  E Yarrow -  Director - Anglo American Exploration (Canada) Ltd

- Mr. R Osmond – Chief geophysicist - Anglo American Exploration (Canada) Ltd

- Mr. C Pretorius – Vice President - GSD

- SPECTREM's Archive

- CONTENTS PAGE

1	SPECTREM SURVEY OF THE LAC BELLEAU AREA	1

1.1	Introduction	1

2	AEM Interpretation of Lac Belleau Area	2

2.1	Geology of the survey area	2

3	Interpretation of AEM anomalies	4

3.1	Zone 1 	5

3.2	Zone 2 	5

3.3	Zone 3 	5

3.4	Zone 4 	5

3.5	Zone 5 	5

3.6	Zone 6 	6

3.7	Zone 7 	6

3.8	Zone 8 	6

4	Appendix 1:  Survey Details	7

4.1.1	Logistics	7

4.1.2	Datum	7

4.1.3	Survey Area Coordinates	7

Appendix 2:  System specifications	8

4.1.4	EM system	8

4.1.5	Magnetic system	8

4.1.6	Positioning system	9

4.1.7	Other sensors	9

5	Appendix 3:  Data processing	10

5.1	Electromagnetic Processing	10

5.1.1	Aircraft Processing	10

5.1.2	Profile data	10

5.1.3	Apparent Conductivity	10

5.1.4	Grids	11

5.2	Magnetic Processing	11

5.2.1	Tie-line leveling	11

5.2.2	Decorrugation	11

5.2.3	Micro-leveling	12

5.3	DEM processing	12

6	Appendix 4:  Deliverables	13

6.1	Digital Products	13

6.1.1	Grids / Profile Data	13

6.1.2	Report	14

6.1.3	Autopick Databases	14

6.2	Maps	14

7	Appendix 5:  Anomaly selection	15

7.1	Electromagnetic Anomaly Selection	15

7.1.1	Conductor Parameterisation and Classification	15

7.1.2	EM Anomaly Grading	16

7.1.3	Complications of Anomaly Interpretation	16

7.1.4	Estimated Conductor Depth	16

8	Appendix 6:  Anomaly Listing	17

8.1	Lac Belleau Area – Anomalies List	18

- List of Figures

Figure 1:  Survey location	1

Figure 2 - An image of the magnetic data	2

Figure 3 - An Image of conductor time constant (Tau Z)	3

Figure 4 - AEM Zone Map	4

# SPECTREM SURVEY OF THE LAC BELLEAU AREA

## Introduction

From 15 September to 17 September 2005, SPECTREM Air Limited conducted airborne electromagnetic, magnetic and radiometric surveys over the Lac Belleau Area in Canada. A total of 325 line kilometres were surveyed.  The general location of the survey is shown in Figure 1.

Figure 1:  Survey location

Details of the survey can be found in Appendix 1.  The system specifications are presented in Appendix 2 and the standard Spectrem Air data processing stream is described in Appendix 3.  A list of the products delivered to the geophysicist in charge of the project is given in Appendix 4.  The criteria and procedures used in anomaly selection are discussed in Appendix 5 and a listing of anomalies is given in Appendix 6.  All the above data is also presented to the client in digital format.

# AEM Interpretation of Lac Belleau Area

## Geology of the survey area

A detailed geological interpretation of the magnetic and electromagnetic data is currently being undertaken by Spectrem Air and will form part of a separate report.

Figure 2 - An image of the magnetic data

Figure 3 - An Image of conductor time constant (Tau Z)

# Interpretation of AEM anomalies

A number of conductive zones are present in the area surveyed.  These are mainly situated within a belt of basalt which extends in a north westerly direction through the area.  The basalt is variably magnetic which indicates that other rock types are probably present within it.  For example some of the more magnetic anomalies in the basalt could be due to ultramafic bodies or sills and these would be of definite interest as ground follow up targets if they had good conductors associated with them.  The data was therefore carefully examined for anomalies of this type.

The conductor dips were generally northwards which probably reflects the regional structure in the area.

Figure 4 - AEM Zone Map

The individual zones selected for ground follow up work will now be discussed in line order.

## Zone 1	

This is a low priority zone situated in wacke near a contact with basalt.  The anomaly on Line 41160 has a high conductance value of 90 Siemens and fairly narrow peak shape which could indicate a localized conductor.  If any ground follow up work is carried out on this zone, the most favourable locality would be on Line 41160, Fiducial 3883, X = 464034, Y = 6829238.

## Zone 2	

A low priority zone situated in basalt.  Both anomalies in the zone have high conductances of 90 Siemens and small associated magnetic anomalies.  The anomaly on Line 41190, Fiducial 5414, X = 464634, Y = 6828405 is the most suitable one for following up on the ground.

## Zone 3	

Low priority has been assigned to this zone which appears to be situated on a contact between basalt and wacke.  The conductor has a high 90 Siemens conductance and its dip is probably fairly flat.  A depth estimate indicates that it could be deep.  The most favourable anomaly in the zone is located on Line 41210, fiducial 6540, X = 465026, Y = 6829209.

## Zone 4	

This is a low priority zone situated in basalt.  The anomaly conductances vary between 42 and 90 Siemens and they all have very small magnetic associations.  Ground follow up work is recommended on Line 41240, Fiducial 8538, X = 465629, Y = 6827966.

## Zone 5	

Although a low priority has been assigned to this zone, it does have some good characteristics which could upgrade it to a higher priority rating.  The zone is situated in basalt immediately south east of Zone 4.  All three anomalies in the zone have conductances of 61 Siemens and fairly narrow anomaly peak shapes.  The magnetic associations vary between 42 and 104 nanoTeslas.  For follow up purposes the best anomaly is located on Line 41260, Fiducial 9728, X = 466032, Y = 6827633.

## Zone 6	

A low priority zone situated in tuff close to a contact with basalt.  All three anomalies in the zone have 61 Siemens conductance values and their magnetic associations vary between 44 and 99 nanoTeslas.  The anomaly located on Line 41360, Fiducial 15523, X = 468021, Y = 6823920 is the most suitable one for following up on the ground.

## Zone 7	

A low priority has been given to this anomaly which is situated in basalt.  Immediately south of the anomaly a magnetic body occurs so it is possible that a conductor may be present here on the contact or at the base of this magnetic body.  The anomaly is located on Line 41370, Fiducial 16331, X = 468234, Y = 6827759.  The conductor may extend at depth on to the adjacent flightlines trending in a south easterly and north westerly direction.

## Zone 8	

A low priority zone situated in tuff and basalt.  Although the conductor appears to be at a deeper depth on Line 41430, it is definitely the more favourable line for initiating ground follow up work.  The conductor here is located on Line 41430, Fiducial 20236, X = 469432, Y = 6825336.

# Appendix 1:  Survey Details

### Logistics

The specific details of the survey were as follows:

| Base of operations | Puvirnituq - Canada |
| --- | --- |
| Flying Dates | Between 15 to 17 September 2005 |
| Survey type | Electromagnetic, magnetic, radiometric, terrain |
| Aircraft type | DC3 – TP67 |
| EM Base Frequency | 45 Hz |
| Nominal aircraft altitude | 90 m |
| Nominal aircraft speed | 60 m/s |
| Acceptable Kilometres flown: | 325 Line kilometres |
| Nominal flight-line spacing | 200 m |
| Nominal flight-line direction: | 0 degrees |
| Nominal tie-line spacing | 2000 m |
| Nominal tie-line direction: | 90degrees |

### Datum

All coordinates provided in this report, in maps and in processed digital data-sets have the following datum parameters.

| Datum | NAD 83 |
| --- | --- |
| Projection | UTM Spheroid : CL66 Zone : 18 |
| Type | Transverse Mercator |
| Dx / Dy / Dx | 4 / 159 / 188 |

### Survey Area Coordinates

The corner coordinates of the survey areas were:

| Area coordinates | Easting (m) | Northing (m) |
| --- | --- | --- |
| LAC BELLEAU | 460933.00 | 6824196.00 |
|  | 460933.00 | 6829966.00 |
|  | 471133.00 | 6829966.00 |
|  | 471133.00 | 6824196.00 |
|  | 460933.00 | 6824196.00 |

# Appendix 2:  System specifications

SPECTREM simultaneously takes electromagnetic, total field magnetic and radiometric measurements.  Both the electromagnetic and magnetic sensors are towed behind the aircraft in “birds” while the radiometric crystals are installed inside the cabin.  The geometry of the system is shown below in Figure 2.  Other system specifications are listed below.

| ### EM system |  |  |
| --- | --- | --- |
| Transmitter height above ground | 91 m |  |
| Tx – Rx vertical separation | 36.6 m |  |
| Tx – Rx horizontal separation | 122.9 m |  |
| Transmitter coil axis | Vertical |  |
| Receiver coil axes | X : horizontal, parallel to flight direction Y : horizontal, perpendicular to flight direction Z : vertical |  |
| Current waveform | Square wave |  |
| Base frequencies | 45 Hz |  |
| Transmitter loop area | 420 m2 |  |
| RMS current | 960 amperes |  |
| RMS dipole moment | 400 000 A.m2 |  |
| Digitising rate @ 45Hz | 46 080 Hz / component |  |
| Recording Rate | 5 Hz |  |
| Number of windows | 8 per component |  |
| Window distribution | Pseudo-binary |  |
| Window Times 45Hz |  |  |
| Window Number | Window Center (s) | Window Width (s) |
| 1 | 21.7 | 21.7 |
| 2 | 54.3 | 43.4 |
| 3 | 119.4 | 86.8 |
| 4 | 249.6 | 173.6 |
| 5 | 501.0 | 347.2 |
| 6 | 1030.8 | 694.4 |
| 7 | 2072.5 | 1388.9 |
| 8 | 4155.8 | 2777.8 |
| ### Magnetic system |  |  |
| Bird height above ground | 72 m |  |
| Bird location | 19 m below and 41 m behind center of aircraft |  |
| Sensor | Scintrex CS-2 Sensor with SPECTREM Counter/Sync System |  |
| Recording Rate | 5 Hz |  |
| Sensitivity | 0.01 nT |  |
| Resolution | 0.1 nT |  |
| ### Positioning system |  |  |
| Sensor | Novatel RT-20 GPS receiver with Fugro Omnistar differential corrections |  |
| Recording Rate | 5 Hz |  |
| ### Other sensors |  |  |
| Radar Altitude | Collins with 5 Hz sampling with 0.3 m resolution |  |
| Laser Altitude | Riegl with 5 Hz sampling with 0.03 m resolution |  |
| Barometric Pressure | Rose Mount with 1 Hz sampling |  |
| Temperature (OAT) | PT-100 RTD with 1 Hz sampling |  |
| Analogue Chart Recorder | RMS GR-33 |  |

# Appendix 3:  Data processing

The EM data was processed in Johannesburg using Geosoft packages.

## Electromagnetic Processing

### Aircraft Processing

Some of the most important EM data processing was carried out on the aircraft as it acquired the data.  The first processing stage was stacking the data to 1024 samples.  The data was then deconvolved to remove system response and transformed to a square wave.  A square transmitter waveform was chosen as a periodic approximation of the step response.

In the next stage of processing the data was binned into 9 channels or windows.  As the SPECTREM system makes its measurement while the transmitter is switched on, it is necessary to separate the primary (transmitted) field from the (induced) secondary field.  The assumption is made that the induced field will have decayed to a minimal amount at the time the last channel is sampled.  As the last channel only measured the primary field, it can be subtracted from the other channels to separate the secondary field.  Hence there are actually 8 channels with geological information in the final data.

### Profile data

The spikes in the line data have been removed using a 3 point Naudy filter.  The line data have also been drift corrected and micro-leveled.  The drift is particularly noticeable on the later time channels and has been applied to channels 4 to 8.  This is an iterative process, with the assumption that there is a constant drift on a single line.  This is reasonable if the lines are short.  The processing steps are:

- The channel data are clipped retaining the data in the resistive areas where the response should be close to zero.

- The average of the clipped data is then calculated and subtracted from the channel data.

The steps are then repeated, refining the correction.

Decorrugation and micro-leveling has been applied to all the channels to reduce small residual errors that have not been corrected through the drift correction method.

### Apparent Conductivity

The apparent conductivity was calculated from its channel amplitude and the aircraft height.  An apparent conductivity is the conductivity of a half space that would produce an amplitude equivalent to the measured response.  It is useful in providing a physically sensible unit and partially compensates for aircraft ground clearance variations.  The unit for apparent conductivity is milliSiemens/meter.

### Grids

The data were gridded using an Akima spline.  System lag was corrected before gridding.

A decorrugation filter was applied to reduce the herringbone effects created by geometrical asymmetry inherent in AEM systems

## Magnetic Processing

The leveling processing included:

- Tie-line leveling

- Decorrugation

- Micro-leveling

### Tie-line leveling

Tie line leveling is used to remove the diurnal variation and errors due to instrument drift, both are assumed to vary slowly over time.

Tie-line leveling is an iterative process:

- Calculate the mis-closures at the crossover points of the tie and traverse lines. The mis-closure is the difference between the magnetic value on the tie line and the traverse line. The mis-closures are weighted by the gradient of the total field at the crossover point.

- The error is approximated by a piecewise polynomial as a function of time along a flight and then along a tie line.

These steps are repeated until a good fit has been obtained.

### Decorrugation

This is a grid based operation designed to reduce the residual errors that the tie-line leveling does not remove. These are due to inaccuracies in the crossovers, localized diurnal activity, and local altitude variations.

Elongated anomalies with the following characteristics are removed:

- 2 times the line spacing perpendicular to the line direction

- 2 times the tie line spacing parallel to the line direction

- small dynamic range

### Micro-leveling

Applies the corrections made to the grid to the profile data and thereby enhances the line data by removing the final residual errors. The micro-leveled data are then gridded.  The lag correction is 40m.

## DEM processing

Initially, the GPS height and the radar altimeter channels are visually inspected and any spikes or discontinuities are removed.  A Low Pass or Naudy Filter is then applied to both channels.  The GPS height channel is then gridded and the resultant grid is checked.  Due to the nature of the GPS data, it is normally necessary at this stage to perform some degree of decorrugation on the grid with the corrections then written back to the database.

The radar altimeter channel is then subtracted from the corrected GPS height channel in the database and the resultant channel is gridded and verified.

# Appendix 4:  Deliverables

## Digital Products

### Grids / Profile Data

(Grids supplied in Geosoft format)

|  | GRIDS (digital) | LINE DATA | MAPS (paper) | PROFILES |
| --- | --- | --- | --- | --- |
| EM Data | 1 | 2 | 3 | 4 |
| EMX1 to EMX8 | Y | Y | - | - |
| EMZ1 to EMZ8. | Y | Y | - | - |
| EMX Apparent Conductivity | - | - | - | - |
| EMZ Apparent Conductivity | - | - | - | - |
| Tau X | Y | - |  | - |
| Tau Z | Y | - |  | - |
| Anomaly Map | Y | Y |  | - |
| Bedrock Elevation X | - | - | - | - |
| Bedrock Elevation Z | - | - | - | - |
| Overburden Thickness X | - | - | - | - |
| Overburden Thickness Z | - | - | - | - |
|  | - | - | - | - |
| TF Magnetic Data |  |  |  |  |
| TFMI | Y | Y |  |  |
| TFMI with IGRF/ DGRF Removed. | - | - | - | - |
| Vertical Derivative. | Y | Y | - | - |
|  | - | - | - | - |
| Terrain |  |  |  |  |
| DEM | Y | Y |  |  |

### Report

- This anomaly selection and logistics report.

### Autopick Databases

- Autopick databases in MS Excel format (copy attached to this report)

## Maps

- AEM Anomalies Interpretation

# Appendix 5:  Anomaly selection

Interpretation of AEM data should follow two approaches, one using profile data for EM anomaly selection and the other using gridded data to produce images for secondary interpretation.

## Electromagnetic Anomaly Selection

The EM profiles were interpreted using the Autopick software developed by SPECTREM.  Anomaly selections were made on the basis of anomaly shape, decay characteristics and magnetic correlation.  Interpreting profile data is important as it contains detail that is lost in the later, grid based, secondary interpretation.

### Conductor Parameterisation and Classification

The EM anomaly interpreter picks and parameterises all EM anomalies of interest in a survey area using a SPECTREM proprietary software suite called Autopick. Using Autopick, the physical location of the electromagnetic conductor can be recorded, and various parameters associated with the conductor can be assigned. These parameters include an anomaly grade, the conductivity-thickness product of the conductor, its mid-time (window 4) residual X channel amplitude, its estimated depth below ground surface, its dip with respect to the nominal survey direction, and the magnitude of its associated magnetic anomaly.

The many anomaly shapes recorded by the SPECTREM electromagnetic system can be classified into three types - cultural, surficial and bedrock.

Cultural conductors are man-made conductors such as fences, power-lines, buried pipes and other metal structures. These give rise to anomalies if they form closed conducting loops, either by being well grounded in a conducting environment, or due to their physical geometry. Cultural conductors can be flagged as such in Autopick in order to reduce the possibility of following these up in the field.

Surficial conductors are flat-lying conductors which occur on or just beneath the ground surface. They generate anomalies which are characteristically broad, of poor conductivity, and large in amplitude. Examples are Quaternary cover and conductive regolith.

Bedrock conductors are typically steeply-dipping narrow zones of high conductivity situated in a relatively resistive host environment. Strike length may be considerable. These conductors present an interpretative problem in selecting from a large number of bedrock anomalies those which are more likely to be due to economic base metal mineralisation.  Anomalies which are seen as more favorable are given a higher grade.

### EM Anomaly Grading

An anomaly grading scheme has been devised to assist in prioritising which anomalies should first be considered for ground follow-up.  This grading scheme is essentially geophysical, being a cumulative assessment by the interpreter of the likelihood of a particular anomaly being a prospective mineral target.  Anomaly grade takes cognisance of such features of the anomaly as its peak shape (width and amplitude), its conductivity-thickness product (CTP) and its magnetic association.

Massive sulphide bodies are usually fairly narrow and of short strike length, and they are often highly conductive.  If sufficient pyrrhotite or magnetite is present, a magnetic anomaly may be associated with the EM anomaly.  Stratiform deposits, however, commonly contain disseminated sulphide mineralisation, and electromagnetic responses over such bodies may be diffuse to almost non-existent.  Strike length can be considerable in the case of stratiform deposits, and magnetic signatures, if any, tend to parallel the trend of the regional geology.

SPECTREM EM anomalies are graded A, B, C or D, with grade A anomalies being the most favorable.

### Complications of Anomaly Interpretation

In the grading process, small, discrete conductors were given a better grade than larger bodies, which were assumed to be lithological.  Lithological conductors are generally formational (i.e. composed of a particular stratigraphic unit), with extended strike lengths, broader anomalies, and moderate to large electromagnetic responses.  Often their conductivity is due to graphitic content rather than economic mineralisation.  However the conclusion should not be drawn that larger conductors are definitely not mineralised.  If mineralisation is disseminated it may have produced a broader, low amplitude EM response.  An ore body’s response may also be masked by nearby lithological or surficial conductors.

The anomaly picking process is used to directly detect an ore body.  This is not the only method through which SPECTREM should be applied.  It is important to remember the geological mapping capabilities of the system, which are covered in the Data Imaging section.

### Estimated Conductor Depth

Caution needs to be taken when using the depth estimates provided in the EM anomaly listings.  Autopick uses as its reference model a 300m by 300m wire loop conductor, which approximates a typical volcanogenic massive sulphide target, but bears very little resemblance to a body of appreciably different dimensions, such as a typical stratiform deposit. For this reason, depth estimates reported by Autopick are unreliable for bodies of dimensions very much greater than 300m by 300m (reported depths are too shallow) and very much less (reported depths are too deep).

# Appendix 6:  Anomaly Listing

These are the EM anomalies interpreted through Autopick.  They are stored digitally in a Microsoft Excel Worksheet stored in the report directory on the CD.  The columns for the anomaly listing are:

| Line # | line number |
| --- | --- |
| Fiducial | fiducial number |
| Lag | lag in fiducials applied to anomaly peak position before plotting |
| Head | heading of line |
| NomH | nominal survey heading |
| UTM X | X coordinate |
| UTM Y | Y coordinate |
| Type | model type, C=culture, ?=possible culture, P=probable culture, N=no culture, |
| CTP | Conductivity thickness product in Siemens |
| X4 | EM window X4 residual amplitude (pp2t, parts per two thousand of the primary field at the receiver) |
| Depth | depth calculated for a 300m X 300m plate with the same response |
| Dip | dip of conductor (degrees) |
| Dip dir | dip direction of conductor |
| Strike | strike of conductor |
| Grade | EM anomaly grade, assigned by interpreter |
| Mag | Residual magnetic anomaly in nT |

## Lac Belleau Area – AnomalY List

Lake Belleau Anomalies.xls