EXECUTIVE SUMMARY

Between May and September 2007, an exploration program was performed on the Marc Bouchard-Gilbert Lamothe and Queenston Mining owned Grizzly property consisting of 2 weeks of prospecting, 1 week of stripping and 10 days of drilling (5 holes).  Results of this exploration program are presented in this report..

Prospecting was conducted inside the Grizzly grid as well as 2.6 km west (Gladstone area), both area included within the limits of Lamark Township (NTS 32G14). Work was mainly concentrated in the vicinity of E-W flowing Daladier creek where carbonatized and mineralized (Py-AsPy) outcrops with anomalous gold values were discovered by prospector Marc Bouchard during a reconnaissance prospecting exercise along a new logging road in June of 2006.

The main purpose of this exploration program was to gain a better understanding of the local geological controls of the mineralization, and hopefully, expose a significant new gold discovery. The second goal of the program was to explain the I.P anomalies detected inside the Grizzly grid. .

## PROPERTY AND LOCATION

The Grizzly Property is held under an option agreement between Queenston Mining Inc. (QMI) and a group of 2 prospectors (Mrs Marc Bouchard and Gilbert Lamothe respectively from Chapais and Arntfield in Québec).

The Property is located approximately 60 kilometers West of the town of Chapais, Québec and lies completely within the townships of Lamark and Guettard (See Fig.1) The Grizzly claim group consists of 86 claim units in three blocs (Main, Keller and Kill blocs) amounting to approximately 4208 hectares (see claim list in  appendix 1) covering about 7 km of N-S stratigraphy and 12 km  of  E-W strike length (see Fig. 2), bordered by the Chibougamau River on the South and the Lac La Trève to the West. The appendix 4 exposed a complete claim list of Grizzly property.

ACCESSIBILITY

Access to the Grizzly Property is accomplished by traveling west from Chapais along Highway 113 for 50 km then north for 20 km along the Poste Abitibi power line road. A 77 km EW logging road (La Bretelle) linking the NS oriented Poste Abitibi road with the NS Barette Nord logging road to the East,  allows access to through  the Grizzly property while networks of recent secondary logging roads provide new access throughout the property.

# CLIMATE, LOCAL RESOURCES, INFRASTRUCTURE AND PHYSIOGRAPHY

The terrain in and around the property comprises numerous small hills and low swampy areas with less than 25 meters of overall relief. Outcrop exposure is generally moderate to excellent with areas of good exposure in both Lamark and Guetard Township. Vegetation on the Grizzly claims is mostly composed of mature spruce and jack pine, growing in moderate to well drained areas.  Deciduous poplars with pine trees are often characteristic of the well drained south-oriented terrain.  A large portion of the area was logged over the past 5 years by ‘Barrette Chapais’, a local lumber company from Chapais.

There are no known environmental liabilities, man-made or natural features that would encumber any future exploration work on the property.  There are however, two native communities (Waswanipi and Oujebougamau) within a 50 km radius.

The Chapais-Chibougamau area offers well-trained exploration and mining personnel.   The Lac Short and the Bachelor gold Mines to the SW are two past operational mines located in the vicinity of the Grizzly property.

Climate conditions are typical of northwestern Québec, with temperatures ranging from   -40 degrees Celsius in the winter to +35 degrees in the summer. Abundant rain and snowfall are usually observed throughout the year.

# HISTORICAL WORKS

- Keller prospect

The Keller prospect (Figure 2) is a block of 4 conventional claim units(16 Ha) located immediately North of the main Grizzly block, and is completely comprised inside the Guêtard township. This prospect was the object of most of the historical exploration effort in the area.

Between the mid-thirties and the late fifties,  The Keller lake prospect was worked by six (6) different exploration companies: Prospectors Airways Limited, Central Chibougamau Mines Limited, Brunswick Quebec Development Limited, Diomar Mining Exploration Syndicate, Fortunata Mines Limited and Power Gold Mines Incorporated, did most of the historical works concerning the Keller prospect.

There is not a complete record of all works done by these different companies. Among the most significant works reported in Kellar Lake area, there are prospecting, sampling, trenching and limited geological mapping. An I.P survey was done by Power Gold Mines in 1976. Two diamond drill holes were drilled by Diomar Mining in 1956. Fortunata Mines Limited drilled few holes in Kellar Lake area but the results were never filed to the Québec Department of Natural Resources. There is no record of exploration work having been done on Kellar prospect from 1958 to 1975.

The last work reported on the Keller prospect was completed by a joint venture comprised of Meston Resources and La Société de Development de la Baie-James (SDBJ) in 1981 (GM 37805). This work consisted of a 14 hole diamond drill program designed to test a 1 km long N-E oriented  I.P. anomaly related to the Kellar structure.

- Other regional work

Eleven (11) diamond drill holes (1 to 11) were drilled in 1956 by Barry Explorations LTD along the Dussault fault,  about 5 kilometers east of the Grizzly prospect. (GM 05274).

Six (6) diamond drill holes (1 to 6) were also drilled in 1956 by Tomiska Copper Mines LTD,  about 2 km north of the Grizzly prospect. Tomiska  tested the Cu-Ni potential of a regional NE diabase dyke present in the area. (GM 07521-A and B)

Opémiska Copper Mines LTD, in the late sixties and early seventies, did a regional drilling program of 24 holes (W-1 to W-22 + 70-L-1 and 70-L-2). These holes were mostly designed to test regional EM anomalies located 3 to 6 kms east of the Grizzly showing  (GM 26458).

Some exploration work including 9 DDH and an Electro-Magnetic (EM) survey were done by Patino Mines (Quebec) LTD in 1976 (GM 33038). Two (2) of these holes        (L-2-1 and L-2-2) were drilled in an area located between 500 and 1000 meters west of the Grizzly showing. These holes were designed to test EM (Input) anomalies for base metal exploration purposes.

In 1988, Exploration Noranda LTÉE did a diamond drill program consisting of 14 holes (LP-88-04 to 17) essentially testing the Blondeau formation,  in an area 2 km south of Keller lake (GM 49083).

Other work in the area was done by Minnova Inc. in 1986-87 (Propriété Rivière Chibougamau (GM 48372)) covering an area north of the Chibougamau River called the “Zee” zone, referring to the point at which the river forms a characteristic “Z” shaped meander. These works (including 4 DDH) were concentrated in Chibougamau River vicinity and along structures related to the proximal Dussault fault were carbonatized shear zones anomalous in Au-As were reported

A geological synthesis including mapping, compilation and new geological interpretation were elaborated by the ‘’Ministère de l’Énergie et des Resources (M.E.R.)’’ using a 2000 km2 cartography program conducted between 1980 and 1982. Townships of Lamark, Guêtard and Julien were part of that new geological interpretation (MM 88-01).

.Recent logging activities in areas have exposed the bedrock in many places, offering new opportunities of prospecting not available to previous prospectors

QUEENSTON 2006 STRIPPING PROGRAM

During the fall of 2006, a trenching program by Queenston Mining exposed a new gold occurrence about 200 meters north of the Daladier creek (figure 2 and 3). The trench revealed the presence of a north trending 1-5 meters wide felsic dyke intruding a gabbroic host which forms part of the 10 kilometers long, EW striking Daladier gabbroic sill.

The mineralization encountered in the trench consisted of variable amounts of AsPy+Py disseminated within strongly carbonatized felsic dyke margins. Assays indicate that gold values are erratic and of marginal grade.

REGIONAL GEOLOGY

The area is underlain by rocks of Archean age of the Gilman and La Trève formations. The rocks in the immediate vicinity of the Grizzly prospect are characterized by the presence of a NE-SW oriented and differentiated gabbroic sill (Daladier sill). This kilometric long mafic intrusion of hundred meters width is located at the interface between the conglomeratic rocks of the La Trève (part of the Daubré) formation to the south, and  the volcanic dominated sequence of the Gilman formation which forms the North portion of the local stratigraphy. (Figure 3) The La Trève formation is a sedimentary sequence that is typically dominated by 80% polymictic conglomerate intercalated with conglomeratic sandstone and meter thick sandstone lenses. Interstratified basaltic flows  are found locally within the La Trève formation.

The rocks of the Gilman formation are dominated by mafic volcanic and related assemblages that are typically composed of basaltic and gabbroic rocks of 10’s to 100’s of meters in thickness. The Gilman stratigraphy is also characterized by presence of volcano-sedimentary interflow tuffs, argillites and other related rocks.

The regional stratigraphy is intruded by 2 major felsic (granite-granodiorite) intrusions (Stock de Guetard (NW) and Stock De La moraine (NE)). An important regional syenitic intrusion (Stock de Saussure) intrudes the Daubré sedimentary sequence 15 km to the SW. Many dykes of felsic composition cut the regional stratigraphy.

Two major regional faults are recognized into the area, the most prominent of which is the regional Lamark fault. This represents a major sub-vertical structure oriented NE-SW and of deca-kilometric size which separates the sedimentary Daubré Formation to the North, from the Waconichi Formation to the south (a sequence of felsic to intermediate tuffaceous and related volcano-clastic rocks).

The second structure, the Dussault fault, is probably a subsidiary of the previous Lamark fault and is oriented NEE-SWW in the property area. This regional fault dips north and divides the northern Daubré (conglomerate dominated) sequences from the Hauy formation (sandstone and conglomerates) to the south.

The regional stratigraphy is oriented ENE, paralleling the main regional schistosity (S2). Regional geological map (Figure 3) suggests the presence of late NE structures dislocating the local stratigraphy.

ECONOMIC GEOLOGY

Many factors point towards an interesting regional potential for gold exploration. The presence of major regional structures such as the Lamark and Dussault faults probably favoured  the circulation of hydrothermal fluids,  potentially gold bearing, at a regional scale. Also the presence of regional felsic intrusions (De La Moraine, Guêtard and Saussure) and their associated dykes may have contributed to hydrothermal fluid circulation  along these regional structures at different periods in the regional geological history.

The past regional gold exploitation at the  Bachelor and Lac Short mines, located in the SW extension of the Lamark fault,  suggests a possible role played in the past by the regional faults in the deposition of gold into these orebodies.

The past exploration programs concerning the Grizzly property, highlighted the presence of anomalous to sub-economic gold occurrences.  As demonstrated by the “Zee zone’’ on the shore of the Chibougamau river and also by the gold occurrence observed in vicinity of the Keller lake to the North, the Grizzly geological setting appears to be favorable for the presence of economic gold deposits.

The presence of Cb-Cl-(Fu) shear zones, anomalous in gold and arsenic, as well as presence of sulfide-gold bearing boulders indicate an interesting gold potential in the area.

2007 PROSPECTING PROGRAM

Between May and July 2007, the Grizzly property was intermittently prospected for a total of 10 days. The prospecting exercise initially targeted anomalies detected by an I.P. survey commissioned by Queenston Mining in the fall of 2006. It was also designed to document the local geology within the limits of the Grizzly Grid along with selected portions of the remainder of the property. A total of 89 rock samples were collected for assay during the prospecting program from 165 outcrops which were examined and their locations fixed with a GPS (Global Positioning System) device (see figure 4).

The 2007 prospection program leeds to the discovery of two (2) new gold occurrences inside the Property limits.  The first gold occurrence (Queenimich prospect) was discovered about 1 km north of the original Grizzly trench (figure 2 and 3). The Queenimich indice returned an initial grab sample of 1.3 g/t Au at the margin of a N240 oriented metric size felsic dyke (Figure 5). A complete list of prospection sample collected is included into appendix 3.

The second gold occurrence (Gladstone) was exposed along a winter logging road about 2.6 km west of Grizzly trench. That new gold occurrence returned an initial gold value up to 11.9 g/t Au inside the mineralized margin of a 10 cm wide N015 oriented Qz-Tourmaline vein in intrusion inside the Daladier gabbroic sill (figure 6).

During August 2007, each of these two (2) new gold sites were further exposed through a mechanical stripping program. The Gladstone prospect was also tested with two short diamond drill holes totaling 198 meters (GRZ-07-04 and GRZ-07-05) both of which revealed the presence of at least 3 distinct parallel ankeritized and mineralized structures. They comprise 1- 5 meter wide sheared/ foliated, NNE trending, gold bearing, alteration zones overprinting a gabbroic section of the Daladier Intrusion  similar to that in the Grizzly trench to the east.

2007 STRIPPING PROGRAM

In August 2007, the two (2) new gold bearing structures discovered earlier in the year through surface prospecting (Queenimich and Gladstone) were uncovered during a 7 day stripping program. Following washing and cleaning of the stripped areas, the exposures were systematically mapped and sampled.

- Queenimich stripping

The Queenimich structure is exposed over an area of about 300 square meters (20 X 15 m.) at UTM coordinates 476750E/5537005N (Zone 18).

Photo 1:  Pillowed basalt confined between two (2) metric size felsic dykes. Basalt appears strongly ankeritized, sericitized and pyritized (AsPy+Py) in vicinity of felsic dyke contacts.

Stripping revealed a simple geological setting consisting of 2 felsic dykes of undetermined thickness (meter size) oriented at N235-N240 and dipping 45-50 degrees NW (figure 5). These dykes intruded a strongly sheared, pillowed and carbonatized basaltic sequence. The carbonatization, shearing and mineralization are mostly confined to the immediate margins of the felsic dykes with mineralization consisting mainly of a combination of disseminated AsPy and Py within a variably sericitized and strongly carbonatized matrix (Photo 1). The Queenimich occurrence shares many similarities with the style of mineralization and alteration observed inside the Grizzly trench located about 1 km south, however, the dyke orientations appear different in both stripped areas.

A total of 20 grab and 91 channel samples (total of 111 samples) were collected from the area of the Queenimich stripping. The best gold value (1.65 g/t Au, Table 1) comes from a grab sample (#22006) comprising 20% AsPy mineralization within the  carbonatized and sheared margin of the southernmost dyke (figure 5). A complete list of Queenimich trench samples is included into appendix 1.

b) Gladstone stripping

The Gladstone prospect stripping, located at UTM 473580E/5535160N (Zone 18) covers about 700 square meters (35 X 20 m.)

Photo 2: N015 decimetric wide Quartz-Tourmaline vein dipping 45o ESE into a strongly ankeritized leucoxenitic gabbro part of the regional Daladier gabbroic sill.

Disseminated Py and minor AsPy mineralization in the Gladstone trench occurs within a medium grained, leucoxenitic gabbro that is strongly ankeritized and  locally fuschite altered (figure 6). Generally, the Gladstone showing consists of a complex Qz-Ak and Qz-Ak-Tm, 1-5 meter wide alteration envelope centered on a 10- 20 cm wide, N015 oriented, quartz-tourmaline vein zone exposed over a length of 20 meters in the stripped area. (Photo 2)

A total of 84 rock samples, 22 grab and 62 channel samples were collected from the area of the Gladstone stripping. The better gold values range between 1.02 g/t Au (sample # 22035) and 11.9 g/t Au (sample # 61176) and are listed in table 2.  A complete list of Gladstone trench samples is included into appendix 2 and 3.

The best gold values were obtained from the mineralized margins of quartz-tourmaline veins. It was noted that veins containing only quartz and ankerite appear to contain lower gold values then those including tourmaline. The vein orientations appear complex and often erratic but a dominant NE direction is inferred. The main vein cuts the gabbro at N015 with a dip of 55 degrees to the ESE.

Two (2) diamond drill holes, GRZ-07-04 and GRZ-07-05 (figures 9 and 10), with collars spaced about 50 meters apart, were drilled in the vicinity of the Gladstone stripping. Both holes revealed the presence of at least 3 parallel, 2-5 meter ankeritic, fuchsitic, and pyritized structures that returned anomalous gold values. From the drilling, it appears that these 3 structures define an alteration corridor at least 50 meters in width and 75 metres along strike. Both the thickness and length of the alteration corridor are open and require additional work to complete the evaluation of the system.

2007 DRILLING PROGRAM

The 2007 drilling campaign on Grizzly property consisted of five (5) holes totaling 831 meters of coring. All diamond drill holes were drilled between August 15 and September 1, and tested different geological and geophysical targets as well as the extension of some of the new gold occurrences discovered during the previous exploration program. All holes successfully explained the various targets tested and all 5 casings were left in place. Hole GRZ-07-05 casing was damaged during the demobilization of the drill. The 5 Grizzly diamond drill hole descriptions are included in appendix 6 and  2007 Assay certificates are grouped into appendix 5.

- GRZ-07-01

Hole GRZ-07-01, collared on section 0+00E / 6+75N (UTM 477200E / 5535447N) at a dip of –48 degrees and azimuth of N150 was designed to test I.P. anomalies with coincident high resistivity and a strong metal factor within a conglomeratic sequence. It intersected a sedimentary package dominated by polygenetic conglomerates interlayered with 2-10 metre thick arkosic, micro-conglomeratic, and, locally, argillitic lenses with all units dipping at 45-50 degrees NW (figure 7). The I.P anomaly was explained by an interval containing up to 30% Py observed in disseminated, clotted, nodular and clastic forms intersected between 63.30 and 64.50 meters. The hole was terminated at 189 meters inside a polygenic conglomeratic unit. No significant gold values were obtained from hole GRZ-07-01.

- GRZ-07-02

Collared on section 0+05W / 0+75N (UTM 476832E / 5536083N) at a dip of –50 degrees and azimuth of N135, hole GRZ-07-02 was designed to serve 3 purposes: 1) testing mineralization below the Grizzly stripping, 2) testing a proximal I.P. anomaly and 3) testing the possibility of the presence of a deformation corridor situated between the  north gabbroic suite (Daladier sill) and the south conglomeratic sequences of La Trève formation. From collar to 52.40 metres, hole GRZ-07-02 traverses the Grizzly trench geology consisting of a leucoxenitic gabbro complex that is locally ankeritized and mineralized with Py and Aspy. The gabbro is cut by several biotitic lamprophyric intrusives and a 1-4 metre wide felsic dyke with the bulk of the mineralization spatially associated with these intrusives. A mineralized, highly magnetic pyroxenitic unit that represents the base of the Daladier gabbroic sill, was intersected between 54.55 and 64.10 meters, explaining the local I.P anomaly detected by the I.P. survey. From 73.30 m. to the end, the hole intersected a sedimentary sequence dominated by intercalated arkose, silstone, argilite and local micro-conglomerate units that are interpreted as part of the base of the La Treve conglomeratic formation. The contact between the northern Daladier gabbroic sill complex and the La Trève sedimentary sequence, appears to be concordant with no evidence of a deformation corridor (figure 8). The core assayed from hole GRZ-07-02 under the Grizzly trench did not return any significant gold values. The best assay (960 ppb Au)  was obtained between 63.60 to 64.10 m. at the base of the pyroxenitic unit.

- GRZ-07-03

Hole GRZ-07-03, laid out to test 2 strong I.P. anomalies within a topographic depression that is located between two gabbroic phases of the Daladier sill, was collared on section 1+00E / 2+50N (UTM 476838E / 5536307N) at a dip of –45 degrees and azimuth of N150.  It was also designed to test the base of the Daladier gabbroic sill about 100 meters east and down dip of the Grizzly stripping.

The first I.P. anomaly appears to represent the expression of a semi-massive (30- 70%), 2.3 metre wide, pyritic and graphitic lens at the upper contact of the  gabbroic sill between 44.7 to 47.0 m. The second I.P. target seems to be explained by a second argilitic unit that is also mineralized with up to 20% disseminated Py over narrow intervals.  In addition, a 27 meter interval within the Daladier sill (114.0 to 137.0 m.) cut a moderately foliated and ankeritized section with up to 2% disseminated Py and local fuchsite alteration. Although hole GRZ-07-03 passed through a thick section of the Daladier gabbroic sill from 82.75 to 200.15 m, the extension of the mineralized pyroxenitic zone observed in previous hole GRZ-07-02 was not identified in GRZ-07-03 (figure 8). No significant gold values were reported in the hole which was terminated at 243.0 meters in the La Treve argilitic, arkosic and conglomeritic formation.

- GRZ-07-04

Hole GRZ-07-04 was collared outside the cut grid at UTM 473617E / 5535215N with a dip of –45 degrees and azimuth of N260 and was designed to test an area under, and 50 metres north, of the Gladstone trench. It revealed the presence of a 70 meter wide, discontinuous deformation corridor including at least 3 separate parallel ankeritic and pyritic structures along the 99 meter length of the hole (figure 9). The first altered corridor (East Gladstone structure) was intersected from 21.7 to 30.0 meters, consisting of a moderate to strong ankeritization with up to 1-2% disseminated Py and local associated fuchsite alteration. The second alteration zone (North extension of Gladstone trench), intersected between 46.0 to 64 .0 meters, consists of moderate pervasive ankeritization with local fuchsite and up to 1-2% disseminated Py. The lowermost interval (West Gladstone ), from 83.20 to 98.0 metres, is similar in intensity to the two previous ones but is mineralized with up to 5% of disseminated Py associated with a 10 and 20cm, brecciated, Qz-Tm- veins with fuchsitic margins intersected between 85.6 and 87.35 meters. Hole GRZ-07-04 was terminated at 99.0 meters in a leucoxenitic gabbro, immediately below the third structure. No significant gold values were encountered in the hole.

- GRZ-07-05

Also collared outside of the grid but at UTM 473620E / 5535164N) at a dip of –45 degrees and azimuth of N260, hole GRZ-07-05 was laid out to test the continuity of mineralization and alteration below the Gladstone stripping. As with previous hole GRZ-07-04, collared 50 meters to the north, hole 05 intersected 3 distinct ankeritic horizons which are located, from top to bottom: from the collar to 8.8 m (Gladstone East), from 21.0 to 44.0 m (Gladstone trench extension), and from 75.0 to 79.0 meters  (Gladstone west, figure 10). Each zone consisted of moderate to strong pervasive ankeritization and partial bleaching of the gabbroic host along with disseminated Py, and local fuchsite alteration associated with Qz-Ak vein margins. Assays from the mineralized zones returned anomalous gold values varying from 310 to 440 ppb in hole GRZ-07-05.

Conclusions

In addition to the previously discovered Grizzly prospect, the 2007 prospecting program was successful in the discovery of two (2) new surface gold occurrences (Queenimich et Gladstone). Both zones were subsequently stripped, washed, mapped and sampled (grab and channel).

The Queenimich stripping highlighted a Grizzly type mineralization characterized by bleached, sericitized, and ankeritized zones that are mineralized with disseminated AsPy at the margins of two, meter wide,  felsic dykes. However, grab and channel samples from both areas (Grizzly and Queenimich), returned only erratic and low grade gold values. For that reason, no further work is proposed in the vicinity of these two stripped exposures.

The Gladstone gold prospect, about 2.6 km west of the Grizzly area, appears more interesting in some aspects such as the frequency and quality of gold values obtained from grab samples as well as apparent presence of a ENE oriented deformation corridor. For these reasons, it is suggested to do additional work in this area including line cutting, I.P. survey and diamond drilling (if new targets are defined by these works).

The 2006 and 2007 exploration program confirms the potential for gold exploration inside the limits of Grizzly property. The low level of exploration committed up to now by Queenston Mining and the rate of discovery of new gold occurrences in the area suggest that a more aggressive exploration program could lead to the discovery of additional new near surface mineralization and hopefully to a possible high grade gold discovery.

RECOMMENDATIONS

For a next exploration program, it is suggested to:

1) 	 Investigate the Gladstone area by geophysics to highlight possible future drill targets in the vicinity.

- Make a closer examination of the Keller prospect in light of the new exposures uncovered by recent logging activity in the vicinity.

.