Mag: Channels

(Oasis Montaj GDB format)

| General line information: |  |  |
| --- | --- | --- |
| Line | Unit | Line number |
| Flt |  | Flight number |
| Date |  | Flight date (yyyy/mm/dd) |
| Clocks and system synchronization: |  |  |
| Fid | Sec | Fiducial |
| GT | Sec | Edited GPS time (second after midnight) |
| Edited GPS channels |  |  |
| X _84 | Metre | Easting, WGS-84 UTM Z18N |
| Y_84 | Metre | Northing, WGS-84 UTM Z18N |
| Zgps | Metre | MSL GPS altitude |
| Longitude | Deg | Longitude, WGS-84 |
| Latitude | Deg | Latitude, WGS-84 |
| Radar altimeter |  |  |
| radarm | Metre | Radar Altimeter |
| DTM | Metre | Digital Terrain Model (ZGPS-radarm-2.0) |
| Ground Mag base station data |  |  |
| Base | nT | Original |
| Basef | nT | Filtered Base |
| Mag TMF data |  |  |
| Magc | nT | Mag despike, (bird sensor) |
| drift | nT | Diurnal correction removed |
| Magbc | nT | Diurnal corrected Mag |
| Corlev | nT | Tie line levelling correction |
| Maglev | nT | Tie line leveled Mag |
| Cormicro | nT | Microleveling correction |
| Magmicro | nT | Micro leveled Mag |

Mag_Otih-Energy_final.grd       Total Magnetic Field grid

V1_ Otih-Energy _final.grd        Total Magnetic Field first vertical derivative grid

V2_ Otih-Energy _final.grd        Total Magnetic Field second vertical derivative grid

Mag_ Otih-Energy _final.gdb    Magnetics database

All grids have a grid size of 25m